Scope of Work

Team Members: Matthew Mendenhall, Gabriel Goeken, Kaleb Mangers

Date Submitted for review: May 8, 2026

Submitted to: Nebraska Department of Transportation

Project Goals:

The goal of this project is to analyze a subset of the Third Generation Simulation (TGSIM) dataset to study agent behavior around intersections. And evaluate the claim that AVs are safer and more predictable than other agents by analyzing 200 seconds of recorded trajectory data. The client has requested two key analyses: (1) Create a quantitative comparison of the behavior of different transportation agents in the 200-second duration data set. And wants to compare acceleration and deceleration patterns, analyze speed profiles, measure and compare average speeds, and detect any behaviors within the data set. (2) Make a visual representation of agent movement at the intersection, either as a static plot of location vs. Time or an animation that highlights how the agents interact over time.

Project Tasks:

Task 1:  Quantitative Behavioral Comparison of Transportation Agents

The consultant will conduct a quantitative comparison of autonomous vehicles (AVs), non-automated passenger vehicles, and pedestrians using 200 seconds of trajectory data from the Foggy Bottom segment of the TGSIM dataset. Because the dataset duration is limited, the analysis will focus on localized interactions and behavior patterns rather than complete trajectory comparisons. The consultant will compare acceleration and deceleration patterns within intersection zones and near pedestrian areas, analyze speed profiles along selected lanes to evaluate consistency and variability between agents, and measure average speeds within intersections. The number and magnitude of deceleration events will be calculated and compared across agent types to identify potentially disruptive behavior. Summary tables and statistical measures will be developed to clearly present and interpret differences among agents, with specific attention given to evaluating whether AVs demonstrate safer and more predictable behavior.

Task 2: Visualization and Reporting of Findings

The consultant will develop clear visualizations to support quantitative analysis, including summary tables, comparative plots, and graphical representations of behavioral metrics. A visual representation of agent movement at the intersection will be created, either as a static plot of spatial position over time or as an animation illustrating how agents interact throughout the 200-second period. All results will be compiled into a structured report that includes a detailed dataset description, explanation of analysis methods, presentation of findings, and interpretation of AV behavior relative to other transportation agents.

Project Deliverables:

Our group will create a GitHub repository of all the files used to create this program. The files that will be included in the GitHub repository are the raw data sheet, the Python code, the written report, the scope of work, and the annotated code document.

Table 1: Final Deliverables

| Deliverable | Files Included | Date |
| --- | --- | --- |
| Raw Data File | CIVE202_Spring2025_Group11_Project5_ | 5/8/2026 |
| Python Code | CIVE202_Spring2025_Group11_Project5_PythonCode | 5/8/2026 |
| Organized Files | GitHub Link containing this data | 5/8/2026 |
| Scope of Work | CIVE202_Spring2025_Group11_Project5_SOW | 5/8/2026 |
| Annotated Code Document | CIVE202_Spring2025_Group11_Project5_ACD | 5/8/2026 |
| Written Report | CIVE202_Spring2025_Group11_Project5_Report | 5/8/2026 |